Rothamsted Research – Risk assessment form

| Description of activity or task (including any tools and equipment to be used) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Preparation and use of reagents for the determination of nitrate/nitrite (3.2.2 and 3.2.3) by colourimetric analysis (RR_QA_Doc/0995/006) |  |  |  |  |  |  |  |  |  |
| Location (including building and room, where applicable. For generic tasks, list the environment, eg labs, offices or outside) |  |  |  |  |  |  |  |  |  |
| Bawden Building, top floor, middle section, room 338 |  |  |  |  |  |  |  |  |  |
| Assessed by |  | Andrea Other |  |  |  |  |  |  |  |
| Date |  | 18.6.15 – updated 18.5.20 |  |  |  |  |  |  |  |
| People who may be affected (numbers) |  |  |  |  |  |  |  |  |  |
| Those directly involved |  |  |  |  | 2 | Members of public |  |  | 0 |
| Others working in vicinity |  |  |  |  | 4 | Young people |  |  | 0 |
| Other staff, eg maintenance and cleaning staff |  |  |  |  | 0 |  |  |  |  |
| Hazards (include amounts and hazard statements for chemicals) |  |  |  | Potential harm (eg injuries or health effects) |  |  |  |  |  |
| Imidazole (< 50 g daily) Danger – Acute toxicity Category 4, serious eye damage and irritation Category 1, reproductive toxicity Category 1B, skin corrosion and irritation Category 1C H318 – Causes serious eye irritation H314 – Causes severe skin burns and eye damage H360D – May damage the unborn child H302 – Harmful if swallowed |  |  |  | P280 – Wear protective gloves, clothing and glasses P301, P330 and P331 – If swallowed, rinse mouth. Do NOT induce vomiting P303, P330 and P353 – If on skin (or hair), take off all contaminated clothing and rinse skin with plenty of water P305, P351 and P338 – If in eyes, rinse cautiously with water for several mins (remove contact lenses if possible) P310 – Immediately call a POISON CENTRE or doctor |  |  |  |  |  |
| Sulphanilamide (< 10 g daily) Warning – Skin corrosion and irritation Category 2, serious eye damage and irritation Category 2, specific target organ toxicity Category 3 H315 – Causes skin irritation H319 – Causes serious eye irritation H335 – May cause respiratory irritation |  |  |  | P280 – Wear protective gloves, clothing and glasses P302 and P352 – If on skin, wash with plenty of water P332 and P313 – If skin irritation occurs, get medical advice P337 and P313 – If eye irritation persists, get medical advice P304 and P340 – If inhaled, remove person to fresh air P312 – Call a POISON CENTRE or doctor if you feel unwell |  |  |  |  |  |
| Naphthylethylene diamine dihydrochloride (< 1 g daily) Warning – Specific target organ toxicity Category 3, skin corrosion and irritation Category 2, serious eye damage and irritation Category 2 H315 – Causes skin irritation H319 – Causes serious eye irritation H335 – May cause respiratory irritation |  |  |  | - P260-Do not breathe dust/fume/gas/mist/vapours/spray. - P280-Wear protective gloves/protective clothing/eye protection/face protection. - P301+P330+P331-IF SWALLOWED: rinse mouth. Do NOT induce vomiting. - P302+P352-IF ON SKIN: Wash with plenty of water/. - P304+P340-IF INHALED: Remove person to fresh air and keep comfortable for breathing. - P305+P351+P338-IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. - P308+P313-IF exposed or concerned: Get medical advice/attention P302 and P352 – If on skin, wash with plenty of water P337 and P313 – If eye irritation persists, get medical advice P304 and P340 – If inhaled, remove person to fresh air P312 – Call a POISON CENTRE or doctor if you feel unwell P280 – Wear protective gloves, clothing and glasses P332 and P313 – If skin irritation occurs, get medical advice |  |  |  |  |  |
| Hydrochloric acid (200 ml daily) Danger – Corrosive to metals Category 1, skin corrosion and irritation Category 1B, serious eye damage and irritation Category 1, specific target organ toxicity Category 3 H290 – May be corrosive to metals H314 – Causes severe skin burns and eye damage H335 – May cause respiratory irritation |  |  |  | P234 – Keep in original container P280 – Wear protective gloves, clothing and glasses P303, P361 and P353 – If on skin (or hair), take off immediately all contaminated clothing. Rinse skin with water or use shower P304 and P340 – If inhaled, remove person to fresh air P305, P351 and P338 – If in eyes, rinse cautiously with water for several mins (remove contact lenses if possible) P310 – Immediately call a POISON CENTRE or doctor |  |  |  |  |  |
| Orthophosphoric acid (200 ml daily) Danger – Skin corrosion and irritation Category 1B, serious eye damage and irritation Category 1, mixtures corrosive to metals Category 1 H290 – May be corrosive to metals H314 – Causes severe skin burns and eye damage |  |  |  | P280 – Wear protective gloves, clothing and glasses P301, P330 and P331 – If swallowed, rinse mouth, do not induce vomiting P303, P361 and P353 – If on skin (or hair), take off immediately all contaminated clothing. Rinse skin with water/shower P305, P351 and P338 – If in eyes, rinse cautiously with water for several mins (remove contact lenses if possible) P310 – Immediately call a POISON CENTRE or doctor |  |  |  |  |  |
| Cadmium (metal granulated) *Purchased in a sealed glass tube – Only handled on breakage Danger – Acute toxicity Category 2, long-term aquatic hazard Chronic 1 and carcinogenicity Category 1B, germ cell mutagenicity Category 2, reproductive toxicity Category 2, specific target organ toxicity Category 1 H372 – Causes damage to organs H400 – Very toxic to aquatic life H350 – May cause cancer H341 – Suspected of causing genetic defects H361fd – Suspected of damaging fertility. Suspected of damaging the unborn child H330 Fatal if inhaled |  |  |  | P201 – Obtain special instructions before use P260 – Do not breathe dust or fumes P273 – Avoid release to the environment P280 – Wear protective gloves, clothing and glasses P304 and P340 – If inhaled, remove person to fresh air |  |  |  |  |  |
| Brij 35 |  |  |  | No hazard |  |  |  |  |  |
| Precautions taken to reduce risk (eg containment or local exhaust ventilation) |  |  |  |  |  |  |  |  |  |
| Suitable protective clothing should be worn when handling harmful, corrosive or toxic substances, ie gloves, lab coats and glasses should be worn when dispensing liquids, which may splash. Small quantities of the toxic substances should be weighed carefully and diluted immediately. For disposal, large crystals should be picked up and dust should be wiped up using a damp cloth, then all stored for disposal in a sealed bag. All acids should be dispensed in a fume cupboard using protective clothing, ie coat, gloves and safety glasses. All waste solutions are collected and sent for specialist disposal. Work to be done by named trained persons only – Wendy Wilmer and Ruth Skilton. |  |  |  |  |  |  |  |  |  |
| Additional documents and information (eg other risk assessments, guidance notes, policies, standard operating procedures or GM project number) |  |  |  |  |  |  |  |  |  |
| Anion lab risk assessment – General Analytical unit SOP 3.2.2 Skalar nitrate chemistry and Skalar 3.2.3 nitrite chemistry as outlined in the analytical protocol and procedure manual. |  |  |  |  |  |  |  |  |  |
| Personal Protective Equipment (PPE) (list type of PPE to be used and any specific requirements) |  |  |  |  |  |  |  |  |  |
| Lab coat, gloves and safety glasses when required. |  |  |  |  |  |  |  |  |  |
| Monitoring procedures (eg health monitoring, machinery or equipment operation and maintenance schedule) |  |  |  |  |  |  |  |  |  |
| Instruments should be maintained regularly, parts kept clean and replaced when necessary. No specific monitoring for chemical use required. |  |  |  |  |  |  |  |  |  |
| Emergency procedures (specialist procedures to adopt in the event of an incident) |  |  |  |  |  |  |  |  |  |
| Spillage procedures Acids – Cover acid spill with Algosol spillage granules, starting at the edge of the spill and working inwards. Allow the granules to soak up the liquid, then scoop up the granules and place in plastic bag for hazardous disposal. First aid is standard – Acid on the skin should be washed in flowing water for 15 minutes. Emergencies: Call reception on emergency ext. 2222, from a mobile phone on 01582 938222. If 2222 is unavailable, phone 999. Others: Call the safety office on ext. 6262 or 07794 697744 (Sam Benson). |  |  |  |  |  |  |  |  |  |
| Is task adequately controlled? |  |  | Yes Hazards arising can be adequately controlled by following good laboratory practice and only fully trained staff are permitted to use this procedure. |  |  |  |  |  |  |
| If “No”, what further action is required to reduce risks? (list actions, by whom and timescale for completion) |  |  |  |  |  |  |  |  |  |
| Signed | A N Other |  |  |  |  |  | Date | (19.6.15) 18.5.20 |  |
| Review date | 18.5.21 |  |  |  |  |  |  |  |  |